# CAN-Sentinel: Rule-Based CAN Intrusion Detection

Contributor: Srivardhni | Role: Member 2 – Detection Rules

Status: Standalone prototype tested locally; integration and simulator validation pending.

# 1. Project context

CAN-Sentinel monitors traffic on a simulated vehicle Controller Area Network (CAN) and flags suspicious activity. ECUs exchange CAN frames containing identifiers and payload bytes. Conventional CAN frames do not inherently authenticate the sender, so a device with network access may inject messages that look like ordinary traffic.

The simulator's configured baseline IDs are 0x100 (vehicle speed), 0x101 (engine RPM), 0x102 (steering angle), and 0x103 (coolant temperature). Kishan confirmed these are standard 11-bit IDs and the simulator uses classic CAN, not CAN FD.

# 2. My responsibility

My responsibility is the rule-based detection module. It accepts a CAN frame record, checks it against configured rules, and returns structured findings. It is designed to remain separate from the traffic pipeline, ML detector, hybrid engine, and dashboard.

Each finding uses the agreed fields: rule_id, title, severity, and reason.

# 3. Implemented checks

Unknown CAN identifier: compares the numeric CAN ID and standard/extended status against configured allowlists. The default standard-ID baseline contains 0x100–0x103. An identifier outside the configured baseline produces an unknown_can_id finding with high severity. An unknown ID is suspicious relative to the simulator baseline; it does not by itself prove an attack.

Payload and frame validation: checks declared DLC, payload length, byte values, required fields, field types, CAN ID ranges, frame flags, and timestamp format. Payload/DLC issues produce invalid_payload_length; malformed records produce malformed_frame; invalid timestamps produce invalid_timestamp. The implementation assumes classic CAN with payloads up to 8 bytes.

Excessive message frequency: tracks timestamps per interface and CAN identifier, distinguishing standard and extended identifiers. It counts frames in a rolling time window and produces a high_message_frequency finding when the configured limit is exceeded. A cooldown reduces repeated alerts during sustained bursts. It handles out-of-order timestamps and stale frames, and reset() clears history between runs.

# 4. Simulator rates confirmed by Kishan

These are target rates for the simulator; observed intervals may vary slightly because of operating-system scheduling. The current default threshold of 20 frames per second is still a placeholder and should be replaced with per-ID thresholds after testing normal traffic.

# 5. Payload encodings confirmed by Kishan

These are prototype-specific simulation encodings, not universal real-vehicle formats.

0x100 speed: two-byte big-endian unsigned value, directly in km/h. Example [0, 72] = 72 km/h.

0x101 engine RPM: two-byte big-endian unsigned value, directly in RPM. Example [12, 248] = 3320 RPM.

0x102 steering angle: two-byte big-endian signed value in tenths of a degree. Example [254, 212] represents -300, or -30.0 degrees.

0x103 coolant temperature: one byte directly in degrees Celsius. Example [89] = 89°C.

The pipeline captures and normalizes payload bytes into frame records; it does not convert them into engineering values. Decoding and signal-aware checks belong in the analysis layer.

# 6. Testing performed

The revised test_detection.py uses Python's built-in unittest framework. The local VS Code terminal output reports 'Ran 105 tests ... OK' on Python 3.13.

Tests cover normal traffic and the finding schema; unknown standard and extended IDs; DLC/payload mismatches; malformed fields and IDs; remote and error frames; invalid and out-of-order timestamps; rolling-window boundaries and cooldown; per-interface counters; configuration; reset behavior; memory limits; and random malformed inputs.

Passing these tests shows the implemented behavior works for the tested cases. It does not establish detection accuracy on realistic vehicle traffic or prove thresholds are calibrated.

# 7. Planned improvements

1. Add expected DLC per ID: 2 bytes for 0x100, 0x101, and 0x102; 1 byte for 0x103.

2. Add signal decoding using the confirmed encodings, then validate plausible values once acceptable ranges are agreed.

3. Replace the single frequency threshold with per-ID thresholds based on normal rates and observed timing variation.

4. Test normal and attack traffic on vcan0; record detections and false positives.

5. Confirm finding schema and new rule IDs with the hybrid-engine and dashboard owners.

6. Integrate with the live pipeline, hybrid engine, and dashboard.

# 8. Limitations and assumptions

The baseline contains four configured IDs; an unknown ID does not automatically mean malicious.

Normal rates are target rates; observed intervals may vary due to scheduling.

The current frequency threshold is not calibrated per ID.

Verify the code before claiming signal decoding and value-range checks are already implemented; these are planned improvements unless they are present in the current file.

Timestamp handling alone does not reliably detect replay attacks.

The module is a tested prototype, not a production-ready automotive IDS. The simulator's encodings should not be generalized to real vehicles.

# 9. Conclusion

A standalone rule-based detector has been implemented and tested locally. It checks configured CAN identifiers, payload/frame consistency, malformed records, timestamps, and excessive message frequency, and returns structured findings. The next stage is to use the confirmed simulator rates and encodings to add expected-DLC and signal-aware checks, calibrate frequency thresholds, test against normal and attack traffic, and integrate with the hybrid engine and dashboard.

## Normal simulator rates

| CAN ID | Signal | Rate | Interval |
| --- | --- | --- | --- |
| 0x100 | Speed | 10 Hz | 100 ms |
| 0x101 | Engine RPM | 10 Hz | 100 ms |
| 0x102 | Steering angle | 20 Hz | 50 ms |
| 0x103 | Coolant temperature | 2 Hz | 500 ms |